## Proposed fitting strategy

Do not fit all CSV cycles directly. First convert the raw data into a clean fitting dataset, then fit the Stanford model in layers.

Your current Python code already groups CSVs by device ID, treats each CSV as one SET–RESET cycle, and outputs plots/summary files, so we can build on that parser instead of starting from zero.

## Best workflow

### 1. Preprocess experimental data

For each folder/condition:

- Parse all CSVs.

- Group by deviceID.

- Split each cycle into:

  - SET branch

  - RESET branch

  - read-region HRS/LRS points

- Detect and reject bad cycles:

  - pure noise

  - no switching

  - open/short-like behavior

  - incomplete voltage sweep

  - extreme current spikes

Output one clean table:

condition, device_id, cycle_id, branch, V, I, quality_score

### 2. Extract compact fitting targets

For each good cycle, extract:

| Target | Purpose |
| --- | --- |
| HRS current at Vread | fit high-gap leakage |
| LRS current at Vread | fit low-gap conduction |
| VSET | fit SET threshold/dynamics |
| VRESET | fit RESET threshold/dynamics |
| full log(I)-V curve | fit overall shape |
| cycle-to-cycle spread | fit variation parameters later |

Use log-current error, not linear current error.

### 3. Fit in stages

#### Stage A — deterministic mean fit

Fit one “representative” curve first:

- median SET curve

- median RESET curve

- one clean device with ~20 cycles

Use Stanford standard model first. The Stanford guide says the standard model is intended for ensemble-average DC switching, while the dynamic model is for fluctuations/variability.

Fit mainly:

gap_ini

gap_min

gap_max

I0

g0

V0

Vel0

beta

gamma0

Rth

The guide lists these as the main device/fitting parameters.

#### Stage B — per-device fitting

After the mean fit works, allow only a few parameters to vary per device:

gap_ini

I0

V0

Vel0

Rth

Keep the rest shared within the same fabrication condition.

#### Stage C — variation fitting

Then turn on the dynamic/variation model and fit:

deltaGap0

T_smth

Goal: match cycle-to-cycle scatter, not just one curve.

## HSPICE automation idea

For every candidate parameter set:

- Generate a temporary .sp deck.

- Use the measured voltage sweep as the input waveform.

- Run HSPICE.

- Parse simulated V/I.

- Interpolate simulation and experiment onto same voltage grid.

- Compute objective score.

- Optimizer proposes next parameters.

The Stanford package already gives DCSweep.sp and pulseSweep.sp templates, and the README notes that SET/RESET require choosing the correct initial HRS/LRS device definition and voltage input.

## Objective function

Use something like:

Loss =

w1 * Huber(log10(|I_exp|) - log10(|I_sim|))

+ w2 * |VSET_exp - VSET_sim|

+ w3 * |VRESET_exp - VRESET_sim|

+ w4 * |log10(I_HRS_exp/I_HRS_sim)|

+ w5 * |log10(I_LRS_exp/I_LRS_sim)|

This prevents the optimizer from only fitting the high-current region.

## Creative but practical idea

Treat fitting as a hierarchical problem:

Fabrication condition parameters

↓

Device-to-device parameter offsets

↓

Cycle-to-cycle stochastic variation

So the final result is not just one parameter set. It becomes:

Condition S1:

mean parameters

device-to-device sigma

cycle-to-cycle sigma

good-cycle yield

That is much more publishable than fitting one IV curve.

## My recommended first milestone

Fit only one clean device first:

1 folder

1 device

best 5–10 clean cycles

standard Stanford model

no variation

log-current objective

Once that works, expand to all devices and then add variation.

## Cycle quality score

The score is only a ranking metric. It does not directly decide accept/reject.

quality_score =

point_count_score

+ voltage_span_score

+ dynamic_range_score

+ SET_presence_score

+ RESET_presence_score

Using current defaults:

| Component | Formula | Max |
| --- | --- | --- |
| Enough points | min(n_points / 40, 2) | 2 |
| Voltage span | min(Vspan / 1.0, 2) | 2 |
| Current dynamic range | min(dynamic_decades / 0.5, 3) | 3 |
| Has SET | +1 | 1 |
| Has RESET | +1 | 1 |

So max score is:

9

## Dynamic range

dynamic_decades = log10( P95(|I|) / P05(|I|) )

Using currents above:

1e-13 A

This checks whether the cycle actually switches or is mostly flat/noise.

## Accept/reject decision

A cycle is accepted only if zero rejection flags are triggered.

Current rejection rules:

| Reject reason | Condition |
| --- | --- |
| too_few_points | less than 40 valid points |
| low_voltage_span | voltage span < 1 V |
| weak_signal | max current < 1 nA |
| too_high_current | max current > 1 A |
| mostly_floor_current | median current < 1e-13 A |
| low_dynamic_range | dynamic range < 0.5 decade |
| missing_set_or_reset | SET or RESET branch not detected |

## SET/RESET detection

A cycle is considered to have SET if:

block title contains "SET"

or max(V) >= 0.5 V

A cycle is considered to have RESET if:

block title contains "RESET"

or min(V) <= -0.5 V

## Important

A cycle can have a decent score but still be rejected.

Example:

good voltage span + many points + dynamic current

but missing RESET

→ rejected

Acceptance is rule-based. Score is for ranking devices/cycles.